| 🔌RFID Pinout and NodeMCU Connection Table |  |  |
| --- | --- | --- |
| RFID Pin; Connects To (NodeMCU Pin); Description. VCC; 3V3; Power. GND; GND; Ground. RST; D3; Reset pin. SDA (SS); D8; Chip Select. MOSI; D7; SPI Data to RFID. MISO; D6; SPI Data from RFID. SCK; D5; SPI Clock. |  |  |

|  | 🖥️ I2C LCD Pinout and NodeMCU Connection Table LCD Pin; Connects To (NodeMCU Pin); Description. VCC; 5V; Power (The LCD module typically needs 5V). GND; GND; Ground. SDA; D2; Serial Data (I2C Data Line, which is GPIO4 on the ESP8266 chip). SCL; D1; Serial Clock (I2C Clock Line, which is GPIO5 on the ESP8266 chip). |  |
| --- | --- | --- |

🔊 Buzzer Pinout and NodeMCU Connection

| Buzzer Pin | Connects To (NodeMCU Pin) | Description |
| --- | --- | --- |
| Positive (+) | D8 (GPIO15) | Signal pin (can use any digital pin) |
| Negative (–) | GND | Common ground |

Flow Chart

Circuite Diagram